Gemini

You said

if I were to make a second unit on a rasberry pi zero again. Can you list for me the things I would need to do for example, to create the correct virtual environment, download libraries, edit configuration, set GPIO pins to correct sense, mange wifi connections. What are the console commands I would need to do this.

Gemini said

Setting up a second unit is a great way to verify your build process. Since the Pi Zero has limited resources, keeping the environment "lean" is key.

Here is the step-by-step console guide to replicating your v39 environment on a fresh Raspberry Pi OS (Lite) installation.

1. System Preparation & Interface Setup

First, ensure the Pi's hardware interfaces (I2C for OLED) are active and the system is up to date.

Bash

# Update system packages

sudo apt update && sudo apt upgrade -y

# Enable I2C and SPI (Required for OLED)

sudo raspi-config nonint do_i2c 0

sudo raspi-config nonint do_spi 0

# Install system-level dependencies for Python and WiFi management

sudo apt install -y python3-pip python3-venv i2c-tools libgpiod-dev nmcli

2. Create the Project Directory & Virtual Environment

Running in a Virtual Environment (venv) protects your system Python from library conflicts.

Bash

# Create project folder

mkdir -p ~/flashair/mirror

cd ~/flashair

# Create the virtual environment

python3 -m venv venv

# Activate it

source venv/bin/activate

3. Install Required Libraries

You need libraries for the OLED, GPIO control, and API requests.

Bash

# Install OLED libraries

pip install luma.oled

# Install HTTP and Request libraries

pip install requests

# Note: 'os', 'json', 'subprocess', 're', 'zipfile', 'io', and 'pathlib'

# are built-in to Python and don't need installation.

4. Configure WiFi Management (nmcli)

Since your script relies on nmcli, you must ensure NetworkManager is handling the WiFi, not the old dhcpcd method.

Bash

# Switch to NetworkManager (Standard on newer Pi OS)

sudo systemctl enable NetworkManager

sudo systemctl start NetworkManager

# Verify your interface is managed

nmcli device

5. Setup GPIO Pins

While the script sets the pins when it runs, you should test the "Sense" of your input pin (GPIO 22) to ensure the pull-up resistor is working.

Bash

# Set Pin 22 as Input with Pull-Up

sudo pinctrl set 22 ip pu

# Check the level (it should say 'hi' when the button is NOT pressed)

pinctrl get 22

6. Configuration & Script Deployment

You will need to manually recreate your config.json file.

Bash

nano ~/flashair/config.json

Paste your configuration:

JSON

{

"flashair_wifi_ssid": "Your_SSID",

"flashair_wifi_password": "Your_Password",

"flashair_ip": "http://192.168.0.1",

"flashair_data_log_dir": "/DATA",

"internet_networks": [

{"ssid": "S21", "password": "xxxx"},

{"ssid": "poohcorner.", "password": "xxxx"}

],

"flysto_email": "your@email.com",

"flysto_password": "yourpassword",

"local_repo_path": "/home/admin/flashair/mirror"

}

7. Automate with Systemd (The Service)

To make the script start on boot, create a service file.

Bash

sudo nano /etc/systemd/system/flysto_sync.service

Paste this content:

Ini, TOML

[Unit]

Description=FlySto Sync Cockpit Service

After=network.target

[Service]

ExecStart=/home/admin/flashair/venv/bin/python /home/admin/flashair/flysto_syncer_v39.py

WorkingDirectory=/home/admin/flashair

StandardOutput=inherit

StandardError=inherit

Restart=always

User=admin

[Install]

WantedBy=multi-user.target

Activate the service:

Bash

sudo systemctl daemon-reload

sudo systemctl enable flysto_sync.service

sudo systemctl start flysto_sync.service

Summary Checklist for the Second Unit:

- Hardware: Solder your Blue (9), White (10), and Green (11) LEDs.

- Connectivity: Ensure the S21 hotspot is in the config.json.

- Paths: Ensure the mirror directory exists so the script doesn't crash on the first download.

Would you like me to provide a "First Boot" test script that just blinks all three LEDs and writes to the OLED to verify your wiring on the second unit?

You said

yes

Gemini said

This "First Boot" diagnostic script is the best way to verify your soldering and I2C wiring before you commit to the full sync logic. It will cycle the OLED and each LED one by one.

1. The Diagnostic Script (test_hardware.py)

Create this file in your project folder:

Python

import os, time, subprocess

from luma.oled.device import ssd1306

from luma.core.interface.serial import i2c

from luma.core.render import canvas

# Initialize Pins

PINS = {'BLUE': 9, 'WHITE': 10, 'GREEN': 11, 'BUTTON': 22}

for name, pin in PINS.items():

if name == 'BUTTON':

os.system(f"sudo pinctrl set {pin} ip pu")

else:

os.system(f"sudo pinctrl set {pin} op dl")

# Initialize OLED

serial = i2c(port=1, address=0x3C)

device = ssd1306(serial, width=128, height=32)

def test_sequence():

print("Starting Hardware Diagnostic...")

with canvas(device) as draw:

draw.text((0, 0), "TESTING LEDS...", fill="white")

for name, pin in PINS.items():

if name == 'BUTTON': continue

print(f"Blinking {name} LED (GPIO {pin})")

os.system(f"sudo pinctrl set {pin} op dh")

time.sleep(2)

os.system(f"sudo pinctrl set {pin} op dl")

print("Testing Button. Press it now...")

with canvas(device) as draw:

draw.text((0, 0), "PRESS BUTTON", fill="white")

timeout = time.time() + 10

while time.time() < timeout:

raw = subprocess.getoutput("pinctrl get 22")

if "level=lo" in raw or "| lo" in raw:

print("Button Pressed!")

with canvas(device) as draw:

draw.text((0, 0), "BUTTON OK!", fill="white")

time.sleep(2)

break

time.sleep(0.1)

with canvas(device) as draw:

draw.text((0, 0), "TEST COMPLETE", fill="white")

print("Hardware verified.")

if __name__ == "__main__":

test_sequence()

2. Full Setup Command Reference

When you are ready to build the real unit, follow this command checklist in order:

Stage A: NetworkManager & System

Bash

# Ensure NetworkManager is active (Standard for v39 logic)

sudo apt install -y network-manager

sudo systemctl enable NetworkManager

sudo systemctl start NetworkManager

Stage B: I2C Permission Fix

If you get a "Permission Denied" error with the OLED, your user needs to be in the i2c group:

Bash

sudo usermod -aG i2c admin

# (Note: Logout and log back in for this to take effect)

Stage C: Verifying "Force Connect" Dependencies

Since the S21 requires a fresh handshake every time, ensure nmcli is fully functional:

Bash

# Test a manual scan to ensure the driver is ready

nmcli device wifi rescan

nmcli device wifi list

Stage D: Directory Permissions

Because the service runs as the admin user, ensure the mirror folder is writable:

Bash

mkdir -p /home/admin/flashair/mirror

sudo chown -R admin:admin /home/admin/flashair

chmod -R 755 /home/admin/flashair

3. Quick Debugging Command Cheat Sheet

Keep these handy for the second unit's maiden flight:

- View live log output: tail -f /home/admin/flashair/sync.log

- Check if the button is physically working: watch -n 0.1 "pinctrl get 22"

- Check OLED I2C address: i2cdetect -y 1 (Should show 3c)

- Restart the sync service: sudo systemctl restart flysto_sync

Would you like me to generate the systemd service file for this unit as well, or are you comfortable adapting the one from the first unit?

You said

yes

Gemini said

Here is the specific systemd service file for your second unit. I have adjusted it to use the Virtual Environment path we established, which is the best practice for a clean installation on a fresh Pi.

1. Create the Service File

Run this command to create the file:

Bash

sudo nano /etc/systemd/system/flysto_sync.service

2. Paste this Configuration

Ini, TOML

[Unit]

Description=FlySto Sync Cockpit Service v39

# Wait for the system to be ready before starting

After=network.target

[Service]

# Points to the python executable inside your virtual environment

ExecStart=/home/admin/flashair/venv/bin/python /home/admin/flashair/flysto_syncer_v39.py

# Ensures the script looks for config.json in the right folder

WorkingDirectory=/home/admin/flashair

StandardOutput=inherit

StandardError=inherit

Restart=always

User=admin

[Install]

WantedBy=multi-user.target

3. Enable and Start

Run these three commands to "set it and forget it":

Bash

# Reload the system to see the new file

sudo systemctl daemon-reload

# Tell the Pi to run this every time it boots

sudo systemctl enable flysto_sync.service

# Start it right now

sudo systemctl start flysto_sync.service

Re-Cap of GPIO Pin "Sensing" for the New Unit

When you wire up the second unit, remember that we chose these specific pins because they are "clean" (they don't have secondary functions like I2C or SPI that might interfere):

- Blue (GPIO 9): BCM 9 / Physical Pin 21

- White (GPIO 10): BCM 10 / Physical Pin 19

- Green (GPIO 11): BCM 11 / Physical Pin 23

- Button (GPIO 22): BCM 22 / Physical Pin 15 (Requires Ground on the other side of the switch)

Final Maintenance Tip: Log Rotation

Since this script writes to sync.log and the Pi Zero has a small SD card, it is a good idea to check the log size occasionally. You can clear it anytime without stopping the service by running:

Bash

truncate -s 0 /home/admin/flashair/sync.log

Your second unit is now ready for the bench test. Would you like me to walk you through how to set up an "Auto-Update" alias so you can push code changes from one Pi to the other over your local network?

My build notes from fist build

pwdFresh raspi install then.

Use imager pi zero 32 bit recomened. Install pi connect with auth token

admin@pizero:// $ sudo apt install python3-dev

admin@pizero:// $ sudo apt install python3-smbus i2c-tools

admin@pizero:// $ sudo apt install python3-pil

admin@pizero:// $ sudo apt install python3-setuptools

admin@pizero:// $ sudo apt install python3-venv

admin@pizero:// $ i2cdetect -y 1

admin@pizero://source venv/bin/activate

Then this one withut sudo

python3 -m pip install --upgrade luma.oled

sudo apt-get update

sudo chown -R $USER:$USER /oled_display/venv  #to set venv permissions so that next command runs ok

(venv) admin@pizero://home/admin $ pip install RPi.GPIO spidev luma.oled

sudo usermod -a -G spi,gpio,i2c $USER

pip install RPLCD smbus2

venv/bin/pip install gpiozero lgpio

edit config.txt file

# Force Button Pull-up and Indicators to High (Off) on boot

gpio=27=ip,pu gpio=9,10,11=op,dh

How to install ‘requests’

Python3 -m venv flystov4_env

Or is it really python3 -m venv venv

Source flystov4/bin/activate

Really its  source venv/bin/activate

(flystov4_env)admin@pizero:~$ pip install requests

For OLE$D09 disply

Install lib

pip3 install adafruit-circuitpython-ssd1306 Pillow

install board, pin

install Adafruit_ssd1306

pip3 install --upgrade adafruit-blinka adafruit-circuitpython-ssd1306

pip install rpi-lgpio

Deactivate to go back to normal CLI

invoke venv by using from root it needs to be started everytime before the program. Giving whole path shows which venv you want to use)

$ home/admin/venv/bin/python /home/admin/helloworld.py  - when you start from the root

Depending where you start from if started from /home/admin the .py filename doesn’t need full path but the env does

So on boot the pi goes to /admin/home then theonly line needed to run is

admin@pizero:~ $ venv/bin/python flystov3.py

admin@pizero:~ $ venv/bin/python shayo_modded.py

gemini suggested better file list script here:

https://share.google/aimode/zSrOYEv40R7P4Qprk

One Final Recommendation

Since this is now working perfectly, you should ensure the script starts automatically when the Pi boots up. If you haven't already, you can add it to the admin user's crontab.

- Run crontab -e.

- Add this line at the bottom:

Bash

@reboot /usr/bin/python3 /home/admin/flashair/orchestrator.py > /home/admin/flashair/sync.log 2>&1

Is there anything else you'd like to tweak—perhaps a specific name for the OLED display or a change to the LED timeout—before you consider this project complete?

To connect a Raspberry Pi Zero to a Windows PC via USB without Wi-Fi, enable USB Ethernet gadget mode by editing the SD card's config.txt and cmdline.txt files. Create a blank ssh file for access, then connect via the inner USB port (data). Use PuTTY to SSH into raspberrypi.local.

YouTube +4

Step-by-Step Guide:

  - Flash SD Card: Use Raspberry Pi Imager to install Raspberry Pi OS (Lite recommended) to your SD card.

  - Enable SSH: Open the boot partition on the SD card in Windows and create an empty file named ssh (no file extension).

  - Configure USB Gadget Mode:

    - Edit config.txt: Open config.txt on the SD card and add dtoverlay=dwc2 to the bottom.

    - Edit cmdline.txt: Open cmdline.txt and add modules-load=dwc2,g_ether after rootwait.

  - Connect USB: Connect the USB cable to the inner Micro USB port (closest to the middle) of the Pi Zero, not the power-only port.

  - Connect to Windows: Plug the other end into your PC. Windows should detect a new USB Ethernet/RNDIS Gadget.

  - SSH into Pi:

    - Open PuTTY or Command Prompt/PowerShell.

    - Connect to: pi@raspberrypi.local.

CLI> sudo nano config.txt

(venv) admin@pizero://oled_display $ python -m pip install --upgrade luma.oled

The program creates the Flasir directory on the pi if its not there already

Flasair

AI Overview

To configure a Toshiba FlashAir W-03 to display all file types (not just photos/DCIM) and allow for general file access, you must manually edit the

CONFIG file in the SD_WLAN directory to enable WebDAV and set the root directory for browsing.

Prerequisites

- Show Hidden Files: The SD_WLAN folder is hidden. In Windows File Explorer, go to the "View" tab and check "Hidden items".

- Text Editor: Use Notepad or Notepad++ to edit the CONFIG file located at F:/SD_WLAN/CONFIG (assuming F: is your card drive).

Steps to Configure for All File Listing

- Open the CONFIG File: Open the SD_WLAN\CONFIG file with a text editor.

- Add/Modify Parameters: Add or change the following lines to enable browsing all files and enable WebDAV (for network drive mapping):

  - UPLOAD=1 (Allows read/write access)

  - UPDIR=/ (Sets the root directory / for browsing, allowing access to files outside DCIM)

  - WEBDAV=2 (Enables WebDAV for network mapping)

  - APPMODE=5 (Optional: Enables Station Mode to connect to your home Wi-Fi instead of its own, if preferred)

- Example CONFIG File:

text

[Vendor]

CIPATH=/DCIM/100__TSB/FA000001.JPG

UPLOAD=1

UPDIR=/

WEBDAV=2

APPMODE=5

... (other existing settings)

- Save and Reinsert: Save the CONFIG file, eject the card, and reinsert it into your device (camera or card reader).

Accessing All Files

- Via Web Browser: Go to http://flashair/ in a browser. The interface will now show the root directory, including non-image files.

- Via WebDAV (Network Drive): In Windows, you can map the card as a network drive to treat it like a local folder. Use \\flashair as the address.

Troubleshooting

- If changes don't apply: The CONFIG file might be protected. You may need to use the FlashAirTool to set the card to read-only, then back to read-write, or ensure UPLOAD=1 is correctly set.

- If you cannot see the files: Ensure UPDIR=/ is used. If it is set to UPDIR=/DCIM, it will only show that folder.

My new config file workin ok

[Vendor]

CIPATH=/DCIM/100__TSB/FA000001.JPG

APPMODE=4

APPAUTOTIME=0

APPNETWORKKEY=********

VERSION=FA9CAW3AW3.00.00

CID=02544d535730384731c786f07900f401

PRODUCT=FlashAir

VENDOR=TOSHIBA

MASTERCODE=f854f6051bcc

LOCK=1

APPINFO=0000000000000000

As bought config

[Vendor]

CIPATH=/DCIM/100__TSB/FA000001.JPG

APPMODE=4

APPNETWORKKEY=********

VERSION=FA9CAW3AW3.00.00

CID=02544d535730384731c786f07900f401

PRODUCT=FlashAir

VENDOR=TOSHIBA

MASTERCODE=f854f6051bcc

LOCK=1

APPINFO=0000000000000000

admin@pizero:~ $ sudo apt update

admin@pizero:~ $ sudo apt install python3-pip

admin@pizero:~ $ python -m venv venv

admin@pizero:~ $ source venv/bin/activate

sudo shutdown -r now

Import "spidev" could not be resolved

The error Import "spidev" could not be resolved occurs because spidev (and RPi.GPIO) are hardware-specific libraries for Linux/Raspberry Pi devices. They cannot be installed or imported natively in a Windows environment (indicated by your file path c:\Git\...).

To fix this for local development on Windows, you should wrap these imports in a try...except block and mock the libraries if they are missing. This allows you to run and test the logic on your PC without crashing, while still working correctly when deployed to the Raspberry Pi.

Note for Deployment

When you deploy this to the actual Raspberry Pi, you will need to ensure the libraries are installed there:

bash

sudo apt-get update

sudo apt-get install python3-spidev python3-rpi.gpio

One Final Recommendation

Since this is now working perfectly, you should ensure the script starts automatically when the Pi boots up. If you haven't already, you can add it to the admin user's crontab.

- Run crontab -e.

- Add this line at the bottom:

@reboot /usr/bin/python3 /home/admin/flashair/orchestrator.py > /home/admin/flashair/sync.log 2>&1

admin@pizero:~ $ crontab -e

no crontab for admin - using an empty one

Select an editor.  To change later, run select-editor again.

1. /bin/nano        <---- easiest

2. /usr/bin/vim.tiny

3. /bin/ed

Choose 1-3 [1]: 1

crontab: installing new crontab

admin@pizero:~ $ shutdown

Failed to schedule shutdown: Interactive authentication required.

admin@pizero:~ $

To run your flysto_syncer_g8kwx_v1.py headless at startup, the most professional and reliable method for a Raspberry Pi is to create a systemd service.

Unlike older methods, systemd will automatically restart your script if it crashes and ensures that all hardware (GPIO, OLED, WiFi) is ready before the script starts.

Step 1: Create the Service File

Open a new service file using the nano editor:

Bash

sudo nano /etc/systemd/system/flysto_sync.service

Step 2: Paste the Configuration

Paste the following block into the editor. This configuration tells Linux to run the script as the admin user but with the permissions needed for hardware access.

Ini, TOML

[Unit]

Description=FlySto Garmin G3X Sync Orchestrator

After=network.target

[Service]

ExecStart=/usr/bin/python3 /home/admin/flashair/flysto_syncer_g8kwx_v1.py

WorkingDirectory=/home/admin/flashair

StandardOutput=inherit

StandardError=inherit

Restart=always

RestartSec=10

User=admin

[Install]

WantedBy=multi-user.target

Step 3: Enable and Start

Run these commands to tell the system about the new service and set it to start at every boot:

- Reload the system: sudo systemctl daemon-reload

- Enable for boot: sudo systemctl enable flysto_sync.service

- Start it now: sudo systemctl start flysto_sync.service

How to Monitor Your Headless Script

Since you won't be seeing the terminal output anymore, you can use these commands to check what the script is doing:

- Check Status: sudo systemctl status flysto_sync.service (This will show you if it is currently running and any recent log lines.)

- View Live Logs: journalctl -u flysto_sync.service -f (This is the equivalent of watching the terminal while the script runs.)

- Stop the Script: sudo systemctl stop flysto_sync.service

Important Note on Paths

Since the script is now running in the background, ensure all paths inside your Python code are Absolute Paths (which we implemented in v36). If you notice the script fails to find config.json, double-check that the WorkingDirectory in the service file matches where your config file is located.

You said

its working fine. One last question, is it possible to make the GPIO lines 9,10,11 be low during he pi boot up (before the cript runs)

Gemini said

That is an excellent catch. By default, Raspberry Pi GPIO pins often float or default to specific states during the first few seconds of boot before the Linux kernel and your Python script take control.

If your Garmin system sees a "Low" (which your level shifter inverts from a High), you might get "ghost" alerts during boot-up. To ensure Pins 9, 10, and 11 stay Low (0V) from the absolute earliest moment possible, we can use the Device Tree Overlay.

The Fix: config.txt Pull-Downs

We can tell the Pi's hardware controller to apply an internal "Pull-Down" resistor to those specific pins at the firmware level. This happens long before your Python script even loads.

- Open the boot configuration file: sudo nano /boot/firmware/config.txt (Note: On older OS versions, this might be /boot/config.txt)

- Scroll to the very bottom and add these lines:

Plaintext

# Ensure Garmin Alert Pins start LOW and stay LOW during boot

gpio=9,10,11=pd,op,dl

  - pd: Sets internal Pull-Down resistors.

  - op: Sets them as Outputs.

  - dl: Forces them to Drive Low ().

- Save and exit (Ctrl+O, Enter, Ctrl+X).

How this changes the behavior

With this change, the electrical timeline looks like this:

| Time | Pin State | Garmin / Level Shifter Result |
| --- | --- | --- |
| Power On | LOW (via Firmware) | Alert OFF (Inverter sees 0V, outputs 12V) |
| Linux Booting | LOW (via Firmware) | Alert OFF |
| Script Starts | LOW (via __init__) | Alert OFF |
| Sync Starts | HIGH (via Python) | Alert ON (Inverter sees 3.3V, outputs 0V) |

Final System Check

| Action | Command |  |
| --- | --- | --- |
| Start it back up | sudo systemctl start flysto_sync.service |  |
| Apply changes | sudo systemctl restart flysto_sync.service |  |
| Check if it's crashed |  | sudo systemctl status flysto_sync.service |  |

sudo systemctl stop flysto_sync.service

  Run sudo systemctl restart flysto_sync.service.

  Press the button and run tail -f /home/admin/flashair/sync.log.